|  |  |  | OPC UA Companion Specification |
| --- | --- | --- | --- |
| OPC 40169-1 |  |  |  |
| OPC UA for Mining External Standards Part 1: IREDES Draft 1.0 2022-10-01 |  |  |  |

| Specification Type: | Industry Standard Specification | Comments: |  |
| --- | --- | --- | --- |
| Doc-Number | OPC 40169-1 |  |  |
| Title: | OPC UA for Mining Part 1 :IREDES | Date: | 2022-10-01 |
| Version: | Draft 1.0 | Software: | MS-Word |
|  |  | Source: | OPC 40169-1-UA_CS_for_Mining-IREDES_RC1.0.docx |
| Author: | IREDES initiative | Status: | Draft |

Revision Protocol

| Version | Date | Description |
| --- | --- | --- |
| 1.0 | October 1st 2022 | First version |

CONTENTS

1	Scope	i

2	Normative references	i

3	Terms, abbreviated terms and conventions	ii

3.1	Overview	ii

3.2	Conventions used in this document	ii

4	General Information to 40169-1: IREDES	3

4.1	introduction to the OPC UA Companion Specification Mining	3

4.2	Introduction to the IREDES standard	3

1	Data Types	4

1.1	IRLengthDataType	4

1.2	DispFlag	4

1.3	Answer	4

1.4	LTPPMptFromType	1

1.1	LTPPMptToType	1

1.6	LTPPMaction	1

1.7	JobAssignmentTimeDataType	6

1.8	Mapping for simple IREDES Data Types	6

6	OPC UA ObjectTypes	7

6.1	ProjectInfoType Object Type	7

7	Profiles and Conformance Units	7

7.1	Conformance Units	7

7.2	Profiles	7

7.2.1	Profile list	7

7.2.2	Server Facets	8

7.2.3	Client Facets	9

8	Namespaces	9

8.1	Namespace Metadata	9

8.2	Handling of OPC UA Namespaces	10

Annex A (normative)   <Title> Namespace and mappings	12

A.1	Namespace and identifiers for <Title> Information Model	12

Figures

Figure 1 – The Scope of OPC UA within an Enterprise	11

Figure 2 – A Basic Object in an OPC UA Address Space	12

Figure 3 – The Relationship between Type Definitions and Instances	13

Figure 4 – Examples of References between Objects	14

Figure 1 – The OPC UA Information Model Notation	14

Tables

Table 11 – Conformance Units for <Title>	9

Table 12 – Profile URIs for <Title>	9

Table 13 - <short name> <Prf1name> Server Profile	9

Table 14 - <short name> <Prf2name> Server Facet	10

Table 11 - <short name> < Prf3name> Client Facet	11

Table 16 – NamespaceMetadata Object for this Document	12

Table 17 – Namespaces used in a <title> Server	12

Table 18 – Namespaces used in this document	12

OPC FOUNDATION, VDMA e.v., Institute for advanced mining technologies and IREDes

____________

This work (OPC UA Companion Specification Template, by OPC Foundation), identified by OPC Foundation, is free of known copyright restrictions.

AGREEMENT OF USE

COPYRIGHT RESTRICTIONS

- This document is provided "as is" by the OPC FOUNDATION, VDMA E.V., INSTITUTE FOR ADVANCED MINING TECHNOLOGIES AND IREDES Right of use for this specification is restricted to this specification and does not grant rights of use for referred documents.

- Right of use for this specification will be granted without cost.

- This document may be distributed through computer systems, printed or copied as long as the content remains unchanged and the document is not modified.

- OPC FOUNDATION, VDMA E.V., INSTITUTE FOR ADVANCED MINING TECHNOLOGIES AND IREDES do not guarantee usability for any purpose and shall not be made liable for any case using the content of this document.

- The user of the document agrees to indemnify OPC FOUNDATION, VDMA E.V., INSTITUTE FOR ADVANCED MINING TECHNOLOGIES AND IREDES and their officers, directors and agents harmless from all demands, claims, actions, losses, damages (including damages from personal injuries), costs and expenses (including attorneys' fees) which are in any way related to activities associated with its use of content from this specification.

- The document shall not be used in conjunction with company advertising, shall not be sold or licensed to any party.

- The intellectual property and copyright is solely owned by the OPC FOUNDATION, VDMA E.V., INSTITUTE FOR ADVANCED MINING TECHNOLOGIES AND IREDES.

PATENTS

The attention of adopters is directed to the possibility that compliance with or adoption of OPC or VDMA e.V. may require use of an invention covered by patent rights. OPC Foundation or VDMA e.V. shall not be responsible for identifying patents for which a license may be required by any OPC or VDMA e.V. specification, or for conducting legal inquiries into the legal validity or scope of those patents that are brought to its attention. OPC or VDMA e.V. specifications are prospective and advisory only. Prospective users are responsible for protecting themselves against liability for infringement of patents.

WARRANTY AND LIABILITY DISCLAIMERS

WHILE THIS PUBLICATION IS BELIEVED TO BE ACCURATE, IT IS PROVIDED "AS IS" AND MAY CONTAIN ERRORS OR MISPRINTS. THE OPC FOUNDATION, VDMA E.V., INSTITUTE FOR ADVANCED MINING TECHNOLOGIES AND IREDES MAKES NO WARRANTY OF ANY KIND, EXPRESSED OR IMPLIED, WITH REGARD TO THIS PUBLICATION, INCLUDING BUT NOT LIMITED TO ANY WARRANTY OF TITLE OR OWNERSHIP, IMPLIED WARRANTY OF MERCHANTABILITY OR WARRANTY OF FITNESS FOR A PARTICULAR PURPOSE OR USE. IN NO EVENT SHALL THE OPC FOUNDATION, VDMA E.V., INSTITUTE FOR ADVANCED MINING TECHNOLOGIES AND IREDES BE LIABLE FOR ERRORS CONTAINED HEREIN OR FOR DIRECT, INDIRECT, INCIDENTAL, SPECIAL, CONSEQUENTIAL, RELIANCE OR COVER DAMAGES, INCLUDING LOSS OF PROFITS, REVENUE, DATA OR USE, INCURRED BY ANY USER OR ANY THIRD PARTY IN CONNECTION WITH THE FURNISHING, PERFORMANCE, OR USE OF THIS MATERIAL, EVEN IF ADVISED OF THE POSSIBILITY OF SUCH DAMAGES.

The entire risk as to the quality and performance of software developed using this specification is borne by you.

RESTRICTED RIGHTS LEGEND

This Specification is provided with Restricted Rights. Use, duplication or disclosure by the U.S. government is subject to restrictions as set forth in (a) this Agreement pursuant to DFARs 227.7202-3(a); (b) subparagraph (c)(1)(i) of the Rights in Technical Data and Computer Software clause at DFARs 212.227-7013; or (c) the Commercial Computer Software Restricted Rights clause at FAR 12.227-19 subdivision (c)(1) and (2), as applicable. Contractor / manufacturer are the OPC Foundation, 16101 N. 82nd Street, Suite 3B, Scottsdale, AZ, 81260-1830

COMPLIANCE

The combination of the OPC FOUNDATION, VDMA E.V., INSTITUTE FOR ADVANCED MINING TECHNOLOGIES AND IREDES shall at all times be the sole entities that may authorize developers, suppliers and sellers of hardware and software to use certification marks, trademarks or other special designations to indicate compliance with these materials as specified within this document. Products developed using this specification may claim compliance or conformance with this specification if and only if the software satisfactorily meets the certification requirements set by the OPC FOUNDATION, VDMA E.V., INSTITUTE FOR ADVANCED MINING TECHNOLOGIES AND IREDES. Products that do not meet these requirements may claim only that the product was based on this specification and must not claim compliance or conformance with this specification.

Trademarks

Most computer and software brand names have trademarks or registered trademarks. The individual trademarks have not been listed here.

GENERAL PROVISIONS

Should any provision of this Agreement be held to be void, invalid, unenforceable or illegal by a court, the validity and enforceability of the other provisions shall not be affected thereby.

This Agreement shall be governed by and construed under the laws of Germany.

This Agreement embodies the entire understanding between the parties with respect to, and supersedes any prior understanding or agreement (oral or written) relating to, this specification.

OPC UA companion specification mining –

40169 Part 1: IREDES

# Scope

This document provides a comprehensive overview of the model information structure of the IREDES standard which in this case uses OPC UA as carrier and is therefore part of the companion specification mining. Version 1.0 of this document covers most of the IREDES type definitions, most of the IREDES ApplicationBaseClasses as well as the LHD truck equipment profile.

# Normative references

The following referenced documents are indispensable for the application of this document. For dated references, only the edition cited applies. For undated references, the latest edition of the referenced document (including any amendments and errata) applies.

OPC 10000-1, OPC Unified Architecture - Part 1: Overview and Concepts

http://www.opcfoundation.org/UA/Part1/

OPC 10000-2, OPC Unified Architecture - Part 2: Security Model

http://www.opcfoundation.org/UA/Part2/

OPC 10000-3, OPC Unified Architecture - Part 3: Address Space Model

http://www.opcfoundation.org/UA/Part3/

OPC 10000-4, OPC Unified Architecture - Part 4: Services

http://www.opcfoundation.org/UA/Part4/

OPC 10000-1, OPC Unified Architecture - Part 1: Information Model

http://www.opcfoundation.org/UA/Part1/

OPC 10000-6, OPC Unified Architecture - Part 6: Mappings

http://www.opcfoundation.org/UA/Part6/

OPC 10000-7, OPC Unified Architecture - Part 7: Profiles

http://www.opcfoundation.org/UA/Part7/

OPC 10000-8, OPC Unified Architecture - Part 8: Data Access

http://www.opcfoundation.org/UA/Part8/

OPC 10000-9, OPC Unified Architecture - Part 9: Alarms and Conditions

http://www.opcfoundation.org/UA/Part9/

OPC 10000-10, OPC Unified Architecture - Part 10: Programs

http://www.opcfoundation.org/UA/Part10/

OPC 10000-11, OPC Unified Architecture - Part 11: Historical Access

http://www.opcfoundation.org/UA/Part11/

OPC 10000-12, OPC Unified Architecture - Part 12: Discovery and Global Services

http://www.opcfoundation.org/UA/Part12/

OPC 10000-13, OPC Unified Architecture - Part 13: Aggregates

http://www.opcfoundation.org/UA/Part13/

OPC 10000-14, OPC Unified Architecture - Part 14: PubSub

http://www.opcfoundation.org/UA/Part14/

OPC 10001-1, OPC Unified Architecture V1.04 - Amendment 1: AnalogItem Types

http://www.opcfoundation.org/UA/Amendment1/

OPC 10001-3, OPC Unified Architecture V1.04 - Amendment 3: Method Metadata

http://www.opcfoundation.org/UA/Amendment3/

OPC 10001-1, OPC Unified Architecture V1.04 - Amendment 1: Dictionary Reference

http://www.opcfoundation.org/UA/Amendment1/

OPC 10001-7, OPC Unified Architecture V1.04 - Amendment 7: Interfaces ad AddIns

http://www.opcfoundation.org/UA/Amendment7/

OPC 10001-11, OPC Unified Architecture V1.04 - Amendment 11: Spatial Types

http://www.opcfoundation.org/UA/Amendment11/

OPC 10000-100, OPC Unified Architecture - Part 100: Devices

http://www.opcfoundation.org/UA/Part100/

OPC 40001-1, OPC Unified Architecture – UA CS for Machinery Part 1 – Basic Building Blocks

http://opcfoundation.org/UA/Machinery/

OPC 40160, OPC Unified Architecture for Mining - General

http://opcfoundation.org/UA/Mining/General/

International Rock Excavation Data Exchange Standard (IREDES) -General Definitions and Standard Architecture
	https://iredes.org

# Terms, abbreviated terms and conventions

## Overview

It is assumed that basic concepts of OPC UA information modelling from OPC 10000-100 and OPC 40001-1 are understood in this document. This document will use these concepts to describe the OPC 40163-1 Information Model. For the purposes of this document, the terms and definitions given in the documents referenced in the OPC Core documents, OPC 10000-10, OPC 40001-1 and OPC 40160 apply.

## Conventions used in this document

The conventions and definitions described in the OPC Core documents, OPC 10000-10, OPC 40001-1 and OPC 40160 apply.

# General Information to 40169-1: IREDES

## introduction to the OPC UA Companion Specification Mining

For general information on the OPC UA Companion Specification Mining and OPC UA in general, please refer to OPC 40160.

As part of the External Standards section of the OPC UA Companion Specification Mining the IREDES standard in the case of this companion specification uses OPC UA as carrier.

## Introduction to the IREDES standard

IREDES is a flexible standard architecture for convenient worksite information exchange in the mining industry. It is used for information exchange between mainly mobile equipment and devices on one side and central computer systems on the other side. Also Machine-to-Machine (“M2M”) communication is covered to a certain extend. Excluded from the standard is any direct communication related to machine remote control as this is inside the product responsibility of the respective suppliers. The standard covers definitions of how to exchange information and what kind of content is exchanged. The flexibility of the standard is achieved by separation between the purpose of information exchange (Application Profile) and the dedicated type equipment using and extending the generic Application Profile information by specific entries for dedicated types of equipment (Equipment Profile).

This principle leads to a matrix structure of the information exchange interface:

- Application Purpose in the Application Profile (vertical structure)

- Equipment specific purposes in the respective Equipment Profile (horizontal structure.

The structure of the IREDES standard is shown in figure 1. The structure is mirrored as far as possible in the IREDES OPC UA information model.

Figure 1 Structure of the IREDES standard

# Data Types

This section contains the definitions of data types used in the IREDES information model.

## IRLengthDataType

This structure contains data describing lengths. It is formally defined in Table 1

Table 1 IRLengthDataType Structure

| Name | Type | Description |
| --- | --- | --- |
| IRLengthDataType | Structure | Subtype of structure defined in OPC UA 1000-3. |
| Value | Double | Data Type for length specification. |
| Unit | EUIinformation | Valid values: mm, cm, m, ft. |

## DispFlag

The enumeration DispFlag is used to determine when to display messages to the machine operator. It is formally defined in Table 2.

Table 2 DispFlag enumeration

| Name | Value | Description |
| --- | --- | --- |
| MachStart | 0 | To be displayed when the machine is started. Machine start is defined as switching on the main power supply or power generation. |
| FileLoad | 1 | To be displayed as soon as the file is loaded (activated) in the machine’s automation system (applicable especially to plan files originating from the mine!). |

## Answer

The following enumeration Answer is used to classify answers to work orders. It is formally defined in

Table 3 Answer enumeration

| Name | Value | Description |
| --- | --- | --- |
| Accepted | 0 | Order is accepted. |
| Delayed | 1 | Order can only be executed with delay. |
| AcceptedWithCondition | 2 |  |
| Denied | 3 | Orders denied. |

## LTPPMptFromType

The enumeration LTPPMptFromType is used to classify mission start points for LHDs. It is formally described in Table 4.

Table 4 LTPPMptFromType Enumeration

| Name | Value | Description |
| --- | --- | --- |
| LoadPt | 0 | Load point |
| DumpPt | 1 | Dump point |
| Parking | 2 | Parking |
| Workshop | 3 | Workshop |
| Others | 4 | Others |

## LTPPMptToType

The enumeration LTPPMptToType is used to classify mission end points for LHDs. It is formally described in Table 1.

Table 1 LTPPMptToType Enumeration

| Name | Value | Description |
| --- | --- | --- |
| LoadPt | 0 | Load point |
| DumpPt | 1 | Dump point |
| Parking | 2 | Parking |
| Boulder | 3 | Boulder |
| Workshop | 4 | Workshop |
| Others | 1 | Others |

## LTPPMaction

The LTPPMaction enumeration is used classify actions carried out at the destination point specified in add reference here. It is formally described in Table 6.

Table 6 LTPPMaction Enumeration

| Name | Value | Description |
| --- | --- | --- |
| Load | 0 | Load |
| Dump | 1 | Dump |
| Parking | 2 | Parking |
| Workshop | 3 | Workshop |
| Other | 4 | Other |

## JobAssignmentTimeDataType

The JobAssignmentTimeDatatype union is used for JobAssignment times. It is formally described in Table 7.

Table 7 JobAssignmentTimeDatatype Union

| Name | Type | Description |
| --- | --- | --- |
| ExpectedFinishTime | 0: DateTime | Time Machine is expected to finish the job. |
| ExpectedDuration | 0:Duration | Time the job is going to take to finish. |

## Mapping for simple IREDES Data Types

Simple IREDES data types will be mapped as described in Table 8.

Table 8 Mapping for simple IREDES data types

| Notation | Data-Type | Value Rank | ArrayDemensions | Description |
| --- | --- | --- | --- | --- |
| IRtextShort | String | -1 | NULL | Text data type element for short text information provided in the IREDES standard. Limited to 24 characters in its original. |
| IRtext | String | -1 | NULL | Text data type for comments or text information provided in the IREDES standard. Originally limited to 64 characters. |
| IRtextLong | String | -1 | NULL | Text data type element for long text information. Limited to 216 characters in the IREDES standard. |
| IRangle | Float | -1 | NULL | Data Type for angle specification. This data type is originally limited to values from -360.000 to +360.000. |
| IRVersion | String | -1 | NULL | REDES Version Numbering. |
| AnyURI | String | -1 | NULL | XMLSimpleType |

# OPC UA ObjectTypes

## ProjectInfoType ObjectType

### Overview

The ProjectInfoType is an ObjectType which contains additional project information. It is based on the ObjectType and intended to be used as AddIn. It’s formal definition can be found in Table 9.

Figure 2 ProjectInfoType Overview

### ObjectType Definition

Table 9 ProjectInfoType ObjectType

| Attribute | Value |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BrowseName | ProjectInfoType |  |  |  |  |  |  |  |
| IsAbstract | False |  |  |  |  |  |  |  |
| References | Node Class | BrowseName |  | DataType | TypeDefinition |  |  | Other |
| Subtype of BaseObjectType as defined in OPC UA 1000-3 |  |  |  |  |  |  |  |  |
| 1: HasProperty | Variable |  | DefaultInstance- BrowseName | String |  | PropertyType |  |  |
| 1: HasComponent | Variable |  | Signature | String |  | BaseDataVariableType | Optional |  |
| 1: HasComponent | Variable |  | Comment | String |  | BaseDataVariableType | Optional |  |

### ObjectType Description

The description can be found in Table 10.

Table 10 ProjectInfoType ObjectType Description

| Component | Description |
| --- | --- |
| Signature | Project signature. |
| Comment | Comments concerning the project can be added here. |

## EquipmentInfoType ObjectType

### Overview

The EquipmentInfoType is an ObjectType which contains equipment specific information concerning the main aggregate the information comes from. ATTENTION: the information shall not be required to interpret the standard conformant data set! It’s just provided to identify a machine. The ObjectType is based on the BaseObjectType and is intended to be used as AddIn.

### ObjectType Definition

Table 11 11 EquipmentInfoType ObjectType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | EquipmentInfoType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Other |
| Subtype of BaseObjectType as defined in OPC UA 1000-3 |  |  |  |  |  |
| 1: HasProperty | Variable | DefaultInstance BrowseName | String | PropertyType |  |
| 1: HasProperty | Variable | EqpManufact | String | PropertyType | Mandatory |
| 1: HasProperty | Variable | EqpType | String | PropertyType | Mandatory |
| 1: HasProperty | Variable | EqpModel | String | PropertyType | Optional |
| 1: HasProperty | Variable | EqpSerNo | String | PropertyType | Optional |
| 1: HasComponent | Variable | EqpSysVer | String | BaseDataVariableType | Optional |
| 1: HasComponent | Variable | EqpInfo | String | BaseDataVariableType | Optional |
| 1: HasComponent | Variable | EqpName | String | BaseDataVariableType | Optional |

### ObjectType Description

The description can be found in Table 12.

Table 12 EquipmentInfoType ObjectType Description

| Component | Description |
| --- | --- |
| EqpManufact | Name of the manufacturer. |
| EqpType | Manufacturer internal type name of the machine. |
| EqpModel | Equipment model describing the model in the specified EqpType. To be stated if required for unequivocal machine type identification. |
| EqpSerNo | Serial number of the machine. |
| EqpSysVer | Version Info Automation System / Software |
| EqpInfo | Other equipment specific information. Free tex |

## GenHeadType ObjectType

### Overview

The GenHeadType ObjectType contains the IREDES General header. It is based on the BaseObjectType and intended to be used as AddIn. It is formally defined in

### ObjectType definition

Table 13 GenHeadType ObjectType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | GenHeadType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Other |
| Subtype of Base Object Type as defined in OPC UA 1000-3 |  |  |  |  |  |
| 1: HasProperty | Variable | DefaultInstance-BrowseName | ‘String | PropertyType |  |
| 1: HasProperty | Variable | FileCreateDate | DateTime | PropertyType | Mandatory |
| 1: HasProperty | Variable | IRVersion | String | PropertyType | Mandatory |
| 1: HasProperty | Variable | DownCompat | String | PropertyType | Mandatory |
| 1: HasAddIn | Object | ProjectInfo | - | Project InfoType | Optional |
| 1: HasAddIn | Object | EquipmentInfo | - | Equipment InfoType | Optional |

### ObjectType Desciption

Table 14 GenHead ObjectType description

| Node | Description |
| --- | --- |
| FileCreateDate | Date of file creation. This is the date/time stamp for initializatioin of the Data Set. |
| IRVersion | Version of the IREDES main components of the standard. this version number changes any time IREDES top level schemas are modified. Please note to state downward compatibility in the separate Attribute. Type definition see below. Fixed 2.0 |
| DownCompat | Downward compatibility of the profile version stated in “version” can be guaranteed down to the version number stated in this attribute. Fixed 2.0 |
| ProjectInfo | Project specific information. Type definition see below. |
| EquipmentInfo | Equipment specific information concerning the main aggregate the information comes from. ATTENTION: This information shall not be required to interpret a standard conformant data set. Type definition see below. |

## DisplayToOperatorType ObjectType

### Overview

The DisplayToOperatorType ObjectType is used to display messages to the operator of a machine. It is based on the BaseObjectType and is intended to be used as AddIn. It’s formal definition is given in Table 11.

### ObjectType definition

Table 11 DisplayToOperatorType ObjectType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | DisplayToOperatorType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Other |
| Subtype of BaseObjectType as defined in OPC UA 1000-3 |  |  |  |  |  |
| 1: HasProperty | Variable | DefaultInstance- BrowseName | String | PropertyType |  |
| 1: Has Component | Variable | DispFlag | Enumeration | BaseDataVariableType | Optional |
| 1: Has Component | Variable | AckFlag | String | BaseDataVariableType | Optional |
| 2: Has Component | Variable | DispText | String | BaseDataVariableType | Optional |

### ObjectType description

| Component | Description |
| --- | --- |
| DispFlag | States under which circumstances the line (message) has to be displayed to the operator |
| AckFlag | Acknowledgement by the operator that he has read the message. (Will be transferred back as soon as the SiteHead is returned to the mine’s computer system with the next protocol exchange. Contains the name of the operator (user name in the Automation system) or simply ACK if automation system does not work with user logins. |
| DispText | Text to be displayed. |

## IROptionType ObjectType

### Overview

The IROptionType is based on the BaseObjectType and is intended to be used as AddIn. Its purpose is to allow individual IREDES standard extensions. Its contents will not be processed. The formal definition is given in Table 16.

### ObjectType definition

Table 16 IROptionType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | IROptionType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Other |
| Subtype of BaseObjectType as defined in OPC UA 1000-3 |  |  |  |  |  |
| 1: HasProperty | Variable | DefaultInstance- BrowseName | String | PropertyType |  |
| 1: HasComponent | Variable | OptionSchema | String | BaseDataVariableType | Optional |

## SiteHeadType ObjectType

### Overview

the SiteHeadType is based on the BaseObjectType and is intended to be used as AddIn. It represents an optional site header. It’s formal definition is given in Table 17.

### ObjectType definition

Table 17 SiteHeadType ObjectType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | SiteHeadType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Other |
| Subtype of BaseObjectType as defined in OPC UA 1000-3 |  |  |  |  |  |
| 1: HasAddIn | Object | DisplayTo Operator | - | DisplayToOperator Type | Mandatory |
| 1: HasAddIn | Object | SiteOption | - | IROptionType | Mandatory |

## GenTrailerType ObjectType

### Overview

The GenTrailerType is based on the BaseObjectType and is intended to be used as AddIn. It contains a CRC 32 checksum which is calculated over the entire IREDES file. This is used for data integrity. The formal definition of the ObjectType is given in Table 18.

### ObjectType definition

Table 18 GenTrailerType ObjectType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | GenTrailerType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Other |
| Subtype of BaseObjectType as defined in OPC UA 1000-3 |  |  |  |  |  |
| 1: HasProperty | Variable | DefaultInstance-BrowseName | String | PropertyType |  |
| 1: HasProperty | Variable | FileCloseDate | DateTime | PropertyType | Mandatory |
| 1: HasProperty | Variable | ChkSum | hexBinary | PropertyType | Mandatory |

## IREDESType ObjectType

### Overview

the IREDESType is part of a complete IREDES data set. It is intended to be used as AddIn and based on the BaseObjectType. It’s formal definition is given in Table 19.

### ObjectType definition

Table 19 IREDESType ObjectType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | IREDESType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Other |
| Subtype of BaseObjectType as defined in OPC UA 1000-3 |  |  |  |  |  |
| 1: HasProperty | Variable | DefaultInstance-BrowseName | String | PropertyType |  |
| 1: HasAddIn | Object | GenHead | - | GenHeadType | Mandatory |
| 1: HasAddIn | Object | SiteHead | - | SiteHeadType | Optional |
| 1: HasProperty | Variable | IRVersion | String | PropertyType | Mandatory |
| 1: HasProperty | Variable | DownCompat | String | PropertyType | Mandatory |

### ObjectType description

Table 20 IREDESType ObjectType Description

| Component | Description |
| --- | --- |
| IRVersion | IREDES Base version needed to process this scheme. |
| IRDownwCompat | Earliest version the IREDES Base system version stated in IRVersion is downward compatible to. Since this version, only extensions have been made but no changes affecting compatibility issues (data type changes etc). |

## OpPerfLogType Object Type

### Overview

The OpPerfLogType Object Type is based on the BaseObjectType ended is intended to be used as AddIn. Its purpose is to accumulate the time of each operation mode during the reporting period. It’s formal definition is given in Table 21.

### OpPerfLogType Object Type Definition

Table 21 OpPerfLogType Object Type Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | OpPerfLogType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Other |
| Subtype of BaseObjectType as defined in OPC UA 1000-3 |  |  |  |  |  |
| 1: HasProperty | Variable | DefaultInstanceBrowseName | String | PropertyType |  |
| 1: HasComponent | Variable | Mworking | Duration | BaseDataVariable Type | Mandatory |
| 1: HasComponent | Variable | WaitProcess | Duration | BaseDataVariable Type | Mandatory |
| 1: HasComponent | Variable | WaitOperator | Duration | BaseDataVariable Type | Mandatory |
| 1: HasComponent | Variable | WaitRepair | Duration | BaseDataVariable Type | Mandatory |
| 1: HasComponent | Variable | WaitSamples | Duration | BaseDataVariable Type | Mandatory |
| 1: HasComponent | Variable | TurnedOff | Duration | BaseDataVariable Type | Mandatory |

### ObjectType Description

Table 22 OpPerfLogType Object Type Description

| Component | Description |
| --- | --- |
| Mworking | Machine working. |
| WaitProcess | Machine waiting for other partners in the process or for process reasons not caused by the machine itself. This may be an (autonomous) machine waiting for access to a shared tramming zone or waiting for access to a dump shaft, a truck to become available etc. See “IREDES Drill Rig profile description” document. |
| WaitOperator | Time the machine waits for operator assistance during the reporting period. See “IREDES Drill Rig profile description” document. |
| WaitRepair | Waiting time for repair until the repair is finished and the machine manually is switched on again. See “IREDES Drill Rig profile description” document. |
| WaitSamples | Waiting time for external supplies like electric power, network connection for remote control (if not in local operation mode), water, material etc. See “IREDES Drill Rig profile description” document. |
| TurnedOff | Machine intentionally put in “OFF” state. This state is only counted if the machine is intentionally deactivated by an operator. Observe that a “switch off” while the machine is in “Wait Repair” mode will be counted as “wait repair” until the machine is switched on again. |

## IRpPerfGenType ObjectType

### Overview

the IRpPerfGenType ObjectType is a generic type used report production performance. It is based on the BaseObjectType and intended to be used as AddIn. It’s formal definition can be found in Table 23.

### ObjectType Definition

Table 23 IRpPerfGenType ObjectType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | IRpPerfGenType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Other |
| Subtype of BaseObjectType as defined in OPC UA 1000-3 |  |  |  |  |  |
| 1: HasProperty | Variable | DefaultInstance-BrowseName | String | PropertyType |  |
| 1: HasAddIn | Object | IREDESType | - | IREDESType | Mandatory |
| 1: HasProperty | Variable | ReportId | String | PropertyType | Mandatory |
| 1: HasProperty | Variable | StartLogTime | DateTime | PropertyType | Mandatory |
| 1: HasProperty | Variable | EndLogTine | DateTime | PropertyType | Mandatory |
| 1: HasComponent | Variable | Comment | String | BaseData VariableType | Optional |
| 1: HasAddIn | Object | OpPerfLog | - | OpPerfLog Type | Optional |

### ObjectType Description

Table 24 IRpPerfGenType Description

| Component | Description |
| --- | --- |
| ReportId | Report id code, to uniquely identify this log report. |
| StartLogTime | Start of the reporting period. Date and time when the first entry to this xml-set was made. |
| EndLogTine | End of the reporting period. Date and time when the last entry to this xml-set was made. |
| Comment | Project information concerning this log. |
| OpPerfLog | Object Type which accumulates the time of each operation mode during the reporting period. |

## IRMaintGenType ObjectType

### Overview

the IRMaintGenType is a generic type for maintenance reporting. It is based on the BaseObjectType and intended to be used as AddIn. It’s formal definition can be found in Table 21.

### ObjectType Definition

Table 21 IRMaintGenType ObjectType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | IRMaintGenType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | Browse Name | DataType | TypeDefinition | Other |
| Subtype of BaseObjectType as defined in OPC UA 1000-3 |  |  |  |  |  |
| 1: HasProperty | Variable | Default-Instance-BrowseName | String |  |  |
| 1: HasAddIn | Object | IREDESType | - | IREDESType | Mandatory |
| 1: HasProperty | Variable | ReportId | String | PropertyType | Mandatory |
| 1: HasProperty | Variable | StartLogTime | DateTime | PropertyType | Mandatory |
| 1: HasProperty | Variable | EndLogTime | DateTime | PropertyType | Mandatory |
| 1: HasComponent | Variable | Comment | String | BaseData VariableType | Optional |

### ObjectType Description

Table 26 IRMaintGenType ObjectType Description

| Component | Description |
| --- | --- |
| ReportId | Report id code, to uniquely identify this log report. |
| StartLogTime | Start of the reporting period. Date and time when the first entry to this xml-set was made. |
| EndLogTime | End of the reporting period. Date and time when the last entry to this xml-set was made. |
| Comment | Project information concerning this log. |

## IRpQualGenType

### Overview

The IRpQualGenType ObjectType is a generic datatype used for production quality reporting. It is based on the BaseObjectType and is intended to be used as AddIn. It’s formal definition can be found in Table 27.

### ObjectType Definition

Table 27 IRpQualGenType ObjectType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | IRpQualGenType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | Browse-+Name | DataType | Type-Definition | Other |
| Subtype of BaseObjectType as defined in OPC UA 1000-3 |  |  |  |  |  |
| 1: HasProperty | Variable | Default-Instance-BrowseName | String | PropertyType |  |
| 1: HasAddIn | Object | IREDESType | - | IREDES Type | Mandatory |
| 1: HasProperty | Variable | ReportId | String | PropertyType | Mandatory |
| 1: HasProperty | Variable | StartLogTime | DateTime | PropertyType | Mandatory |
| 1: HasProperty | Variable | EndLogTime | DateTime | PropertyType | Mandatory |
| 1: HasComonent | Variable | Comment | String | BaseData VariableType | Optional |
| 1: HasComonent | Variable | OperatorId | String | BaseData VariableType | Optional |
| 1: HasComonent | Variable | PlanIdRef | String | BaseData VariableType | Optional |
| 1: HasComonent | Variable | PlanNameRef | String | BaseData VariableType | Optional |

### ObjectType Description

Table 28 IRpQualGenType ObjectType Description

| Component | Description |
| --- | --- |
| ReportId | Report id code, to uniquely identify this log report. |
| StartLogTime | Start of the reporting period. Date and time when the first entry to this xml-set was made. |
| EndLogTine | End of the reporting period. Date and time when the last entry to this xml-set was made. |
| Comment | Project information concerning this log. |
| OperatorId | Identify the operator of the machine for reference. |
| PlanIdRef | Reference to plan ID if the quality report relates to a working plan. The value is identical to the ID value stated in the object DRPPlan/DrillPosPlan/PlanID. |
| PlanNameRef | Reference to plan logical name if the quality report relates to a working plan. The value is identical to the ID value stated in the object DRPPlan/DrillPosPlan/PlaName. |

## IRplanGenType ObjectType

### Overview

The IRplanGenType ObjectType is based on the BaseObjectType and intended to be used as AddIn. Its purpose is to provide a generic datatype for production planning. It’s formal definition can be found in Table 29.

### ObjectType Definition

Table 29 IRplanGenType ObjectType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | IRplanGenType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | Browse Name | DataType | TypeDefinition | Other |
| Subtype of BaseObjectType as defined in OPC UA 1000-3 |  |  |  |  |  |
| 1: HasProperty | Variable | Default-Instance-BrowseName | String | PropertyType |  |
| 1: HasAddIn | Object | IREDES Type | - | IREDESType | Mandatory |
| 1: HasProperty | Variable | PlanName | String | PropertyType | Mandatory |
| 1: HasProperty | Variable | PlanId | String | PropertyType | Mandatory |
| 1: HasComonent | Variable | Comment | String [8] | BaseDataVariable Type | Optional |
| 1: HasComonent | Variable | Project | String | BaseDataVariable Type | Optional |
| 1: HasComonent | Variable | WorkOrder | String | BaseDataVariable Type | Optional |

### ObjectType Description

Table 30 IRplanGenType ObjectType Description

| Component | Description |
| --- | --- |
| PlanId | IREDES internal production plan ID used for reference e.g. by Production Quality data sets basing on a particular production plan. |
| PlanName | Plan logical name to identify this specific plan to the human user. Useful to help the operator of a machine to logical identify a specific plan. |
| Comment | Comments to the plan for example type of plan, purpose, tools to use. |
| Project | Project id code. To identify the target project for this plan. |
| WorkOrder | Work order id code. To identify the work order associated with this plan. |

## IRStatusGenType

### Overview

The IRStatusGenType is based on the BaseObjectType and intended to be used as AddIn. The IRStatusGenType is a generic datatype for status reporting. It’s formal definition can be found in Table 31.

### ObjectType Definition

Table 31 IRStatusGenType ObjectType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | IRStatusGenType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | Browse-Name | DataType | Type-Definition | Other |
| Subtype of BaseObjectType as defined in OPC UA 1000-3 |  |  |  |  |  |
| 1: HasProperty | Variable | Default-Instance-BrowseName | String | PropertyType |  |
| 1: HasAddIn | Object | IREDESType | - | IREDESType | Mandatory |
| 1: HasProperty | Variable | ReportId | String | PropertyType | Mandatory |
| 1: HasProperty | Variable | StartLogTime | DateTime | PropertyType | Mandatory |
| 1: HasProperty | Variable | EndLogTime | DateTime | PropertyType | Mandatory |
| 1: HasComonent | Variable | Comment | String | BaseData VariableType | Optional |
| 1: HasComonent | Variable | OperatorId | String | BaseData VariableType | Optional |

### ObjectType Description

Table 32 IRStatusGenType ObjectType Description

| Component | Description |
| --- | --- |
| ReportId | Report id code, to uniquely identify this log report. |
| StartLogTime | Start of the reporting period. Date and time when the first entry to this xml-set was made. |
| EndLogTine | End of the reporting period. Date and time when the last entry to this xml-set was made. |
| Comment | Project information concerning this log. |
| OperatorId | Identify the operator of the machine for reference. |

## IRContourBaseType ObjectType

### Overview

The IRContourBaseType which is used as a contour ID datatype. It is based on the BaseObjectType and intended to be used as AddIn. It’s formal definition is given in Table 33.

### ObjectType Definition

Table 33 IRContourBaseType ObjectType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | IRContourBaseType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | BrowseName | Data- Type | Type Definition | Other |
| Subtype of BaseObjectType as defined in OPC UA 1000-3 |  |  |  |  |  |
| 1: HasProperty | Variable | DefaultInstance-BrowseName | String | PropertyType |  |
| 1: HasProperty | Variable | ContourId | String | PropertyType | Mandatory |
| 1: HasComonent | Variable | ContourType | String | BaseData VariableType | Optional |
| 1: HasComonent | Variable | ContourLeftWidth | Float | BaseData VariableType | Optional |
| 1: HasComonent | Variable | ContourRightWidth | Float | BaseData VariableType | Optional |

### ObjectType Description

Table 34 IRContourBaseType ObjectType Description

| Component | Description |
| --- | --- |
| ContourId | A contour Id which points to a IREDES contour definition. |
| ContourType | Type of contour definition. |
| ContourLeftWidth | Distance from PointCoord to the left most coordinate of the Contour, regarded from tunnel direction. For graphic presentation purpose. |
| ContourRightWidth | Distance from PointCoord to the right most coordinate of the Contour, regarded from tunnel direction. For graphic presentation purpose. |

## IRdrillPlanBaseType ObjectType

### Overview

The IRdrillPlanBaseType is a generic datatype used for drill planning. It is based on the BaseObjectType and intended to be used as AddIn. It’s formal definition can be found in Table 31.

### ObjectType Definition

Table 31 IRdrillPlanBaseType ObjectType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | IRdrillPlanBaseType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | Browse-Name | DataType | Type-Definition | Other |
| Subtype of BaseObjectType as defined in OPC UA 1000-3 |  |  |  |  |  |
| 1: HasProperty | Variable | Default-Instance-BrowseName | String | PropertyType |  |
| 1: HasProperty | Variable | DrillPlanId | String | PropertyType | Mandatory |
| 1: HasComonent | Variable | DrillPlanType | String | BaseData VariableType | Optional |
| 1: HasComonent | Variable | DrillPlanLeft Width | Float | BaseData VariableType | Optional |
| 1: HasComonent | Variable | DrillPlanRightWidth | Float | BaseData VariableType | Optional |

### ObjectType Description

Table 36 IRdrillPlanBaseType ObjectType Description

| Component | Description |
| --- | --- |
| DrillPlanId | A drill plan Id which points to a DRPPlan(IREDES Drill Rig Production Plan). |
| DrillPlanType | Type of drill plan (Bolt, Injection, Cut etc.). |
| DrillPlanLeft Width | Distance from PointCoord to the left most coordinate of the Contour, regarded from tunnel direction. For graphic presentation purpose. |
| DrillPlanRightWidth | Distance from PointCoord to the right most coordinate of the Contour, regarded from tunnel direction. For graphic presentation purpose. |

## IRRealTimeGenType ObjectType

### Overview

The IRRealTimeGenType ObjectType is a generic real-time data type. It is based on the BaseObjectType and intended to be used as AddIn. It’s formal definition can be found in Table 37.

### ObjectType Definition

Table 37 IRRealTimeGenType ObjectType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | IRRealTimeGenType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | BrowseName | DataType | Type Definition | Other |
| Subtype of BaseObjectType as defined in OPC UA 1000-3 |  |  |  |  |  |
| 1: HasProperty | Variable | DefaultInstance-BrowseName | String | PropertyType |  |
| 1: HasAddIn | Object | IREDESType | - | IREDESType | Mandatory |
| 1: HasProperty | Variable | ReportTimeStamp | DateTime | PropertyType | Mandatory |
| 1: HasComonent | Variable | Comment | String [10] | BaseData VariableType | Optional |

### ObjectType Description

Table 38 IRRealTimeGenType ObjectType Description

| Component | Description |
| --- | --- |
| ReportTimeStamp | Timestamp indicating when the report is composed. |
| Comment | Further information concerning this log. |

## IRWorkOrderReplyGenType

### Overview

The IRWorkOrderReplyGenType ObjectType is used for replies to work orders. It is based on the BaseObjectType and intended to be used as AddIn. It’s formal definition can be found in Table 39.

### ObjectType Definition

Table 39 IRWorkOrderReplyGenType ObjectType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | IRWorkOrderReplyGenType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | BrowseName | DataType | Type Definition | Other |
| Subtype of BaseObjectType as defined in OPC UA 1000-3 |  |  |  |  |  |
| 1: HasProperty | Variable | DefaultInstance-BrowseName | String | PropertyType |  |
| 1: HasComponent | Variable | Answer | Answer | BaseData VariableType | Mandatory |
| 1: HasComponent | Variable | Duration | Duration | BaseData VariableType | Optional |
| 1: HasComponent | Variable | Condition | String | BaseData VariableType | Optional |
| 1: HasComponent | Variable | Extra | String [10] | BaseData VariableType | Optional |
| 1: HasComponent | Variable | Reason | String[1] | BaseData VariableType | Optional |

### ObjectType Description

Table 40 IRWorkOrderReplyGenType ObjectType Description

| Component | Description |
| --- | --- |
| Answer | See Answer enumeration. |
| Duration | for delayed option only. |
| Condition | only for "AcceptedWithCondition" option. Free text field for condition such as "need cosumable", etc. |
| Extra |  |
| Reason | place to clarify reasons for delaying or denying the work order. |

## IRreplyType

### Overview

The IRreplyType ObjectType combines IRWorkOrderReplyGenType and an accompanying comment in one type. The IRreplyType is based on the BaseObjectType and is intended to be used as AddIn. It’s formal description can be found in Table 41.

### ObjectType Definition

Table 41 IRreplyType ObjectType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | IRreplyType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | Browse Name | DataType | Type Definition | Other |
| Subtype of BaseObjectType as defined in OPC UA 1000-3 |  |  |  |  |  |
| 1: HasProperty | Variable | Default-Instance-Browse-Name | String | PropertyType |  |
| 1: HasAddIn | Object | Reply | - | IRWorkOrderReply GenType | Mandatory |
| 1: HasComponent | Variable | Comment | String[1] | BaseDataVariable Type | Mandatory |

## IRWorkOrderGenType ¶

### Overview

The IRWorkOrderGenType is based on the BaseObjectType and intended to be used as AddIn. It’s used for work assignments to a machine.  It’s formal definition can be found in Table 42.

### ObjectType Definition

Table 42 IRWorkOrderGenType ObjectType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | IRWorkOrderGenType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | BrowseName | DataType | Type Definition | Other |
| Subtype of BaseObjectType as defined in OPC UA 1000-3 |  |  |  |  |  |
| 1: HasProperty | Variable | DefaultInstance-BrowseName | String | PropertyType |  |
| 1: HasAddIn | Object | IREDESType | - | IREDESType | Man-datory |
| 1: HasComponent | Variable | AssignmentTime | JobAssign-mentTimeDataType | BaseData VariableType | Man-datory |
| 1: HasComponent | Variable | WorkOrderType | String | BaseData VariableType | Optional |
| 1: HasComponent | Variable | PriorityLevel | UInt32 | BaseData VariableType | Man-datory |
| 1: HasProperty | Variable | WorkOrderId | String | Property Type | Man-datory |
| 1: HasProperty | Variable | Issuer | String | Property Type | Man-datory |
| 1: HasComponent | Variable | machineId | String [10] | BaseData VariableType | Optional |
| 1: HasComponent | Variable | operatorId | String[10] | BaseData VariableType | Optional |
| 1: HasAddIn | Object | WorkOrderContent | - | IROption Type | Man-datory |
| 1: HasComponent | Variable | SafetyAdvice | String[10] | BaseData VariableType | Optional |
| 1: HasComponent | Variable | SpecialCondition | String[10] | BaseData VariableType | Optional |
| 1: HasComponent | Variable | SpecialTask | String[10] | BaseData VariableType | Optional |
| 1: HasComponent | Variable | Comment | String [1] | BaseData VariableType | Optional |
| 1: HasAddIn | Object | Reply1 | - | IRreplyType | Optional |
| 1: HasAddIn | Object | Reply2 | - | IRreplyType | Optional |
| 1: HasAddIn | Object | Reply3 | - | IRreplyType | Optional |
| 1: HasAddIn | Object | Reply4 | - | IRreplyType | Optional |
| 1: HasAddIn | Object | Reply1 | - | IRreplyType | Optional |
| 1: HasAddIn | Object | Reply6 | - | IRreplyType | Optional |
| 1: HasAddIn | Object | Reply7 | - | IRreplyType | Optional |
| 1: HasAddIn | Object | Reply8 | - | IRreplyType | Optional |
| 1: HasAddIn | Object | Reply9 | - | IRreplyType | Optional |
| 1: HasAddIn | Object | Reply10 | - | IRreplyType | Optional |

### ObjectType Description

Table 43 IRWorkOrderGenType ObjectType Description

| Component | Description |
| --- | --- |
| AssignmentTime | Time the execution of the job is expected to take. |
| WorkOrderType |  |
| PriorityLevel | Priority of the workorder. Level from 1 to 10 as 10 representing top priority. |
| WorkOrderId | Identification of this work order. This id has nothing to do with the, for example, work plan id. |
| Issuer | ID of the person or institute who issued this work order. |
| MachineId | Machine assigned to the work. |
| OperatorId | Operators assigned to the work. |
| WorkOrderContent | General work order description. |
| SafetyAdvice | Work order shall be carried out. schema chould be defined separately within the IRoptionType. |
| SpecialCondition | e.g. electricity will be switched off in some region in mine during operation. |
| SpecialTask | Tasks that are not or hard to describe in workorder. |
| Comment | Further information |
| Reply 1-10 | Work order reply. |

## IRLTMMonType

### Overview

The IRLTMMonType ObjectType is based on the BaseObjectType and is intended to be used as AddIn. The IRLTMMonType is used for LHD machine monitoring reporting. It’s formal definition can be found in Table 44.

### ObjectType Definition

Table 44 IRLTMMonType ObjectType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | IRLTMMonType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | BrowseName | DataType | Type Definition | Other |
| Subtype of BaseObjectType as defined in OPC UA 1000-3 |  |  |  |  |  |
| 1: HasProperty | Variable | DefaultInstance-BrowseName | String | PropertyType |  |
| 1: HasAddIn | Object | IREDESType | - | IREDESType | Mandatory |
| 1: HasAddIn | Object | GenTrailerType | - | GenTrailer Type | Mandatory |
| 1: HasProperty | Variable | LTMMonVersion | String | PropertyType | Mandatory |
| 1: HasProperty | Variable | LTMMonDownw Compat | String | PropertyType | Mandatory |

### ObjectType Description

Table 41 ObjectType Description

| Property | Description |
| --- | --- |
| LTMMonVersion | Fixed V 1.0 |
| LTMMonDownwCompat | Fixed V 1.0 |

## IRLTPlanType ObjectType

### Overview

The IRLTPlanType is used for LHD production planning. It is based on the BaseObjectType and intended to be used as AddIn. It’s formal definition can be found in Table 46.

### ObjectType Definition

Table 46 IRLTPlanType ObjectType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | IRLTPlanType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | BrowseName | Data-Type | Type Definition | Other |
| Subtype of BaseObjectType as defined in OPC UA 1000-3 |  |  |  |  |  |
| 1: HasProperty | Variable | DefaultInstance-BrowseName | String | PropertyType |  |
| 1: HasAddIn | Object | IRplanGenType | - | IRplanGenType | Mandatory |
| 1: HasAddIn | Object | GenTrailerType | - | GenTrailerType | Mandatory |
| 1: HasProperty | Variable | LTPlanVersion | String | PropertyType | Mandatory |
| 1: HasProperty | Variable | LTPlanDownw-Compat | String | PropertyType | Mandatory |

### ObjectType Description

Table 47 IRLTPlanType ObjectType Description

| Component | Description |
| --- | --- |
| LTPlanVersion | Fixed V 1.0 |
| LTPlanDownwCompat | Fixed V 1.0 |

## LTPPwaitProcType

### Overview

The LTPPwaitProcType ObjectType is used to document process caused waiting time. It is based on the BaseObjectType and intended to be used as AddIn. It’s formal definition can be found in Table 48.

### ObjectType Definition

Table 48 LTPPwaitProcType ObjectType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | LTPPwaitProcType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | Browse Name | Data-Type | Type Definition | Other |
| Subtype of BaseObjectType as defined in OPC UA 1000-3 |  |  |  |  |  |
| 1: HasProperty | Variable | Default-Instance-Browse-Name | String | PropertyType |  |
| 1: HasComponent | Variable | BlastDelay | DateTime | BaseDataVariableType | Optional |
| 1: HasComponent | Variable | CantDump | DateTime | BaseDataVariableType | Optional |
| 1: HasComponent | Variable | Traffic | DateTime | BaseDataVariableType | Optional |
| 1: HasComponent | Variable | NoRock | DateTime | BaseDataVariableType | Optional |
| 1: HasComponent | Variable | MineUtils | DateTime | BaseDataVariableType | Optional |
| 1: HasComponent | Variable | Rework | DateTime | BaseDataVariableType | Optional |
| 1: HasComponent | Variable | RoadMaint | DateTime | BaseDataVariableType | Optional |

### ObjectType Description

| Component | Description |
| --- | --- |
| BlastDelay | Any Delay caused by blasting operations. |
| CantDump | Dump point blocked by another machine, boulders or dump shaft filled / truck missing. |
| Traffic | Traffic caused delays: roadway blocked by another machine / cars / other traffic. |
| NoRock | Wait for material to handle - No access to material to load. |
| MineUtils | Waiting for mine utilities. |
| Rework |  |
| RoadMaint | Waiting for roadway maintenance. |

## LTPPTimeRepType

### Overview

The LTPPTimeRepType is used for time reporting concerning a particular load/dump point pair. It is based on the BaseObjectType and intended to be used as an AddIn. It’s formal definition can be found in Table 49.

### ObjectType Definition

Table 49 LTPPTimeRepType ObjectType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | LTPPTimeRepType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | BrowseName | DataType | Type Definition | Other |
| Subtype of BaseObjectType as defined in OPC UA 1000-3 |  |  |  |  |  |
| 1: HasProperty | Variable | DefaultInstanceBrowseName | String | PropertyType |  |
| 1: HasProperty | Variable | LTPPStartTime | UtcTime | PropertyType | Mandatory |
| 1: HasProperty | Variable | LTPPEndTime | UtcTime | PropertyType | Mandatory |

## LTPPMissionType

### Overview

The LTPPMissionType is used to generate a report for each tramming order ("mission") run during the reporting period. One object per mission! One "mission" is defined as tramming from A to B. It is based on the BaseObjectType and intended to be used as AddIn. It’s formal definition can be found in Table 10.

### ObjectType Definition

Table 10 LTPPMissionType ObjectType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | LTPPMissionType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | BrowseName | DataType | Type Definition | Other |
| Subtype of BaseObjectType as defined in OPC UA 1000-3 |  |  |  |  |  |
| 1: HasProperty | Variable | DefaultInstance-BrowseName | String | PropertyType |  |
| 1: HasComponent | Variable | LTPPMisSeq | UInt64 | BaseData-VariableType | Mandatory |
| 1: HasComponent | Variable | LTPPMptFromN | String | BaseData-VariableType | Mandatory |
| 1: HasComponent | Variable | LTPPMptFrom-ID | String | BaseData-VariableType | Optional |
| 1: Has-Component | Variable | LTPPMptFrom-Type | LTPPMpt-FromType | BaseData-VariableType | Optional |
| 1: HasComponent | Variable | LTPPMptToN | String | BaseData-VariableType | Mandatory |
| 1: HasComponent | Variable | LTPPMptToID | String | BaseData-VariableType | Optional |
| 1: Has-Component | Variable | LTPPMptTo-Type | LTPPMptTo Type | BaseData-VariableType | Optional |
| 1: Has-Component | Variable | LTPPMarea | String | BaseData-VariableType | Optional |
| 1: HasProperty | Variable | LTPPMisstart | DateTime | PropertyType | Mandatory |
| 1: HasProperty | Variable | LTPPMissEnd | DateTime | PropertyType | Mandatory |
| 1: HasComponent | Variable | LTPPMwaitPoint | DateTime | BaseData-VariableType | Optional |
| 1: Has-Component | Variable | LTPPMaction | LTPPMaction | BaseData-VariableType | Mandatory |
| 1: Has-Component | Variable | LTPPMwaitgen | DateTime | BaseData VariableType | Optional |
| 1: HasComponent | Variable | LTPPMtimeAct | DateTime | BaseData-VariableType | Optional |
| 1: HasComponent | Variable | LTPPMpayld | Float | BaseData-VariableType | Mandatory |
| 1: HasComponent | Variable | LTPPMpayldQ | String | BaseData-VariableType | Optional |
| 1: HasProperty | Variable | LTPPMtramEnd | DateTime | PropertyType | Mandatory |
| 1: HasComponent | Variable | LTPPMtramDist | Float | BaseData-VariableType | Mandatory |
| 1: HasComponent | Variable | LTPPMopID | String | BaseData-VariableType | Optional |

### ObjectType Description

Table 11 LTPPMissionType ObjectType Description

| Component | Description |
| --- | --- |
| LTPPMisSeq | Sequence number of the mission. Starting at 1 with the first mission in the reporting period. |
| LTPPMptFromN | Name of the point where the mission originated (tramming started). |
| LTPPMptFromID | Electronic (Tag) ID of the point where the mission originated (Tramming started). Electronic ID of the point stated in LTPPMptFromN. |
| LTPPMptFromType | Type of the point where the mission started. |
| LTPPMptToN | Name of the destination point, where the tramming finished and the mission ended. |
| LTPPMptToID | Electronic (tag) ID of the point where the mission ended (destination point). |
| LTPPMptToType | Type of the point where the mission ended. |
| LTPPMarea | ID for the mine area the machine is operating in. Usually both departure and destination points should be located in this area. |
| LTPPMisstart | Time tag when the mission started. |
| LTPPMissEnd | End time of the mission. Counting ends when the machine is ready to start the next mission, including all waiting before the next mission can be started. |
| LTPPMwaitPoint | Waiting time for destination point availability. |
| LTPPMaction | Action to be carried out at destination point specified in LTPPMptTo. |
| LTPPMwaitgen | Accumulated waiting time during the mission, excluding the time reported in LTPPMwaitPoint. |
| LTPPMtimeAct | Duration of the action carried out at the destination point in LTPPMaction. |
| LTPPMpayld | Tonnage of payload carried between start and destination points. |
| LTPPMpayldQ | Payload quality indicator. - Data type preliminarily set to text! |
| LTPPMtramEnd | Tramming end time: Time stamp when the tramming ended at destination point. |
| LTPPMtramDist | Tramming distance between start and destination point. Unit: km; Resolution: 0.0001km (10cm). |
| LTPPMopID | Operator ID |

## LTPPaccPtsType

### Overview

The LTPPaccPtsType is used to document data for each pair of load point / dump point. It is based on the BaseObjectType and intended to be used as AddIn. It’s formal definition can be found in Table 12.

### ObjectType Definition

Table 12 LTPPaccPtsType ObjectType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | LTPPaccPtsType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | BrowseName | DataType | Type Definition | Other |
| Subtype of BaseObjectType as defined in OPC UA 1000-3 |  |  |  |  |  |
| 1: HasProperty | Variable | LTPPLdrawPtN | String | PropertyType | Mandatory |
| 1: HasComponent | Variable | LTPPLdrawPtID | String | BaseData VariableType | Optional |
| 1: HasProperty | Variable | LTPPLdumpPtN | String | PropertyType | Mandatory |
| 1: HasComponent | Variable | LTPPLdumpPtID | String | BaseData VariableType | Optional |
| 1: HasComponent | Variable | LTPPLmass | Float | BaseData VariableType | Mandatory |
| 1: HasComponent | Variable | LTPPLcycl | UInt16 | BaseData VariableType | Mandatory |
| 1: HasComponent | Variable | LTPPLdist | Float | BaseData VariableType | Mandatory |
| 1: HasComponent | Variable | LTPPLopObserv | String | BaseData VariableType | Optional |
| 1: HasAddIn | Object | LTPPTimeRep | - | LTPPTime-RepType | Optional |

### ObjectType Description

| Component | Description |
| --- | --- |
| LTPPLdrawPtN | Name of the draw (load point) accessed in the reported job. |
| LTPPLdrawPtID | Electronic (tag) ID of the draw (load point) in this combination (Name in 1.1.1.1). |
| LTPPLdumpPtN | Name of the dump point in this combination. |
| LTPPLdumpPtID | Electronic (tag) ID of the draw (load point) in this combination (Name in 1.1.1.1). |
| LTPPLmass | Mass transported between this point pair during reporting period in t. Min accuracy: 0.01t. |
| LTPPLcycl | Number of cycles travelled between this point pair during reporting period. |
| LTPPLdist | Distance travelled between those two points during reporting period. Accumulated distance of al rounds travelled. Both routes are counted! Accuracy: 0,01 km |
| LTPPLopObserv | Operator observations regarding the travelway, load or dump points during reporting period (e.g. loose rock, bad roadway,... Preliminarily a string, later we can add preselect-lists for easier operator input! |
| LTPPTimeRep | Time reporting for access to the particular load / Dump point pair. Multiple elements may be required as work can be interrupted and restarted again at a later time during reporting period. |

## LTPPLoadRepType

### Overview

The LTPPLoadRepType is a special type used for LHD load reporting. It is based on the BaseObjectType and intended to be used as AddIn. It’s formal definition can be found in Table 13.

### ObjectType Definition

Table 13 LTPPLoadRepType ObjectType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | LTPPLoadRepType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | Browse-Name | Data Type | Type Definition | Other |
| Subtype of BaseObjectType as defined in OPC UA 1000-3 |  |  |  |  |  |
| 1: HasProperty | Variable | Default-Instance-BrowseName | String | PropertyType |  |
| 1: HasComponent | Variable | LTPPCyclTot | UInt64 | BaseDataVariableType | Mandatory |
| 1: HasComponent | Variable | LTPPdistTot | Float | BaseDataVariableType | Optional |
| 1: HasComponent | Variable | LTPPwrkDist | Float | BaseDataVariableType | Optional |
| 1: HasComponent | Variable | LTPPloadTot | Float | BaseDataVariableType | Mandatory |
| 1: HasAddIn | Object | LTPPwaitProc | - | LTPPwaitProcType | Optional |
| 1: HasAddIn | Object | LTPPaccPts | - | LTPPaccPtsType | Optional |
| 1: HasAddIn | Object | LTPPMission | - | LTPPMissionType | Optional |

### ObjectType Description

Table 14 LTPPLoadRepType  ObjectType Description

| Component | Description |
| --- | --- |
| LTPPCyclTot | Total number of working cycles (rounds) completed during the reporting period |
| LTPPdistTot | Overall distance travelled in during the reporting period. This includes also non-performance related tramming e.g. to workshop,... Minimum accuracy required by the standard:: 0,1 km |
| LTPPwrkDist | Total distance travelled in a working mode (as reported by MWorking) during the reporting period. |
| LTPPloadTot | Total load carried under all completed working cycles during reporting period. Minimum accuracy required by the standard: 0,01 |
| LTPPwaitProc | Process caused waiting time - LHD specific! Specifies details of the WaitProc timing in the Application Profile! |
| LTPPaccPts | Report data for each pair of load point / dump point. |
| LTPPMission | See LTPPMissionType. |

## IRLTPPerfType

### Overview

The IRLTPPerfType ObjectType is used for LHD production performance reporting. It is based on the BaseObjectType and intended to be used as AddIn. It’s formal definition is given in Table 11.

### ObjectType Definition

Table 11 IRLTPPerfType ObjectType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | IRLTPPerfType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | BrowseName | Data Type | Type Definition | Other |
| Subtype of BaseObjectType as defined in OPC UA 1000-3 |  |  |  |  |  |
| 1: HasProperty | Variable | DefaultInstance-BrowseName | String | PropertyType |  |
| 1: HasAddIn | Object | IRpPerfGenType | - | IRpPerfGen-Type | Mandatory |
| 1: HasAddIn | Object | LTPPLoadRep-Type | - | LTPPLoadRep-Type | Mandatory |
| 1:HasProperty | Variable | LTPPerfVersion | String | PropertyType | Mandatory |
| 1: HasProperty | Variable | LTPPerfDownw-Compat | String | PropertyType | Mandatory |

### ObjectType Description

| Component | Description |
| --- | --- |
| LTPPLoadRep | Reports on how much material has been transported between load and dump points during the reporting period. |
| LTPPerfVersion | Fixed V 1.0 |
| LTPPerfDownwCompat | Fixed V 1.0 |

## IREDES LHDTruckType ObjectType

### Overview

The IREDES LHDTruckType ObjectType is intended to be used as AddIn or for instantiation. It constitutes a complete IREDES LHDTruck equipment profile. It is based on the BaseObjectType. It’s formal definition can be found in Table 16.

### ObjectType Definition

Table 16 IREDES LHDTruckType ObjectType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | IredesLhdTruckType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | BrowseName | Data Type | Type Definition | Other |
| Subtype of BaseObjectType as defined in OPC UA 1000-3 |  |  |  |  |  |
| 1: HasProperty | Variable | DefaultInstanceBrowseName | String | PropertyType |  |
| 1: HasAddIn | Object | IRLTPPerfType | - | IRLTPPerfType | Mandatory |
| 1: HasAddIn | Object | IRLTPlanType | - | IRLTPlanType | Mandatory |
| 1: HasAddIn | Object | IRLTMMonType | - | IRLTMMonType | Mandatory |

# Profiles and Conformance Units

Profiles and ConformanceUnits break functionality into testable groups. All companion specification shall include at least one Profile/Facet. If there are any groupings of functionality that not all Servers/Client would implement then multiple Profile/Facet are encouraged. A ConformanceUnit should describe a testable unit. A single ConformanceUnit is tested as a unit so all items covered by it must be support or the ConformanceUnit will fail. ConformanceUnits can be included in multiple Profiles, thus they are declared in their own table.

The name of the Profile should end with Facet or Profile. A Facet is a grouping of functionality that must also be paired with other Facets to create a running Server or Client. A Profile is all inclusive, in that is the Profile is implemented no additional functionality would be required to have a running application.

<short name>

A <short name> is required for each companion specification to assure uniqueness of string identifiers. It precedes the names of Profiles and ConformanceUnits and is included in URIs and URLs defined in a companion specification.

A <short name> is all caps if an acronym, otherwise camel case.

Exception if the short name is a trademark. Use trademark casing.

## Conformance Units

Table 34 defines the corresponding ConformanceUnits for the OPC UA Information Model for <title>.

Table 17 – Conformance Units for <Title>

| Category | Title | Description |
| --- | --- | --- |
| Server | <short name> <Function1> | Supports the base functionality defined in <Title> Information Model. This includes….. |
| Server | <short name> <Function2> | Supports the …... |
| Server | <short name> <Function3> | Supports the …... |
| Client | <short name> Client <Function1> | The client can make use of the …... |

Typically, Client ConformanceUnits describe the use of a function, but they do not need to match 1 to 1 with Server ConformanceUnits.  They might also reference to other categories defined in Part 7 (Pub, Sub, GDS…). For larger companion specifications, there might be separate tables for Client ConformanceUnits, Server ConformanceUnits, etc.

## Profiles

### Profile list

Table 31 lists all Profiles defined in this document and defines their URIs.

Table 18 – Profile URIs for <Title>

| Profile | URI |
| --- | --- |
| <short name> <Prf1name> Server Profile | http://opcfoundation.org/UA-Profile/<short name>/Server/<Prf1name> |
| <short name> <Prf2name> Server Facet | http://opcfoundation.org/UA-Profile/<short name>/Server/<Prf2name> |
| <short name> <Prf3name> Client Facet | http://opcfoundation.org/UA-Profile/<short name>/Client/<Prf3name> |

### Server Facets

#### Overview

The following sections specify the Facets available for Servers that implement the <title> companion specification.  Each section defines and describes a Facet or Profile.

A specification can define multiple Facets if not all features are to be implemented by all Servers and Clients. The name of the Facet shall give a hint of the subset. An overall description shall be provided that explains the subset and it potential use.

#### <short name> <Prf1name> Server Profile

Table 36 defines a Profile that describes the …….

Table 19 - <short name> <Prf1name> Server Profile

| Group | Conformance Unit / Profile Title | Mandatory / Optional |
| --- | --- | --- |
| Profile | 0:Core 2017 Server Facet http://opcfoundation.org/UA-Profile/Server/Core2017Facet |  |
| Profile | 0:UA-TCP UA-SC UA Binary http://opcfoundation.org/UA-Profile/Transport/uatcp-uasc-uabinary |  |
| Profile | 0:Data Access Server Facet http://opcfoundation.org/UA-Profile/Server/DataAccess |  |
| Profile | 2:BaseDevice_Server_Facet |  |
| Profile | <short name> <Prf2name> Server Facet |  |
| Subscription Services | 0:Subscription Durable | M |
| <short name> | <short name> <Function1> | M |

This table lists a Profile, in which it includes other base Profiles that would be needed to make a working Server.  It also includes other Facets defined in this companion specification and ConformanceUnits defined in this companion standard.

A namespace shall be included if Profiles or ConformanceUnits of another specification are included. In the example above '0' represents the OPC UA core specification and '2' UA for Devices (see Table 41).

The column with title "Mandatory / Optional" defines whether support of included ConformanceUnits is optional or mandatory. Optional means that an application has the option to not support the ConformanceUnit. However, if supported, the application shall pass all tests associated with the ConformanceUnit.

The "Group" for all ConformanceUnits defined in this document shall be the <short name>. If ConformanceUnits of OPC 10000-7 are referenced, the corresponding Groups shall be used. See the example with group "Subscription Services".

#### <short name><Prf2name> Server Facet

Table 37 defines a Facet that describes the …….

Table 60 - <short name> <Prf2name> Server Facet

| Group | Conformance Unit / Profile Title | Mandatory / Optional |
| --- | --- | --- |
| <short name> | <short name> <Function1> | M |
| <short name> | <short name> <Function3> | O |

This table lists a Facet, in that it must be include with other Facets to create a running application.  It defines the ConformanceUnits and other facets that are required

### Client Facets

#### Overview

The following tables specify the Facets available for Clients that implement the <title> companion specification.

A specification can define multiple facets if not all features are to be implemented by all Servers and Clients. The name of the facet shall give a hint of the subset. An overall description shall be provided that explains the subset and it potential use.

#### <short name> < Prf3name> Client Facet

Table 38 defines a Facet that describes the base characteristics for all OPC UA Clients that make use of this companion specification. Additional Profiles will define support for various information models that are part of this document.

Table 61 - <short name> < Prf3name> Client Facet

| Group | Conformance Unit / Profile Title | Mandatory / Optional |
| --- | --- | --- |
| Profile | 0:AddressSpace Lookup Client Facet http://opcfoundation.org/UA-Profile/Client/AddressSpaceLookup |  |
| Profile | 0:DataAccess Client Facet http://opcfoundation.org/UA-Profile/Client/DataAccess |  |
| Profile | 0:DataChange Subscriber Client Facet http://opcfoundation.org/UA-Profile/Client/DataChangeSubscriber |  |
| Session Services | 0:Session Client Detect Shutdown | M |
| <short name> | <short name> Client <Function1> | M |

This table lists a Facet, in that it must be include with other Facets to create a running application.  It defines the ConformanceUnits and other facets that are required as an example it include other base Facets and a Base system ConformanceUnit

# Namespaces

## Namespace Metadata

Namespace Metadata are required for any companion standard that specifies an information model (e.g. Objects and ObjectTypes). The metadata provide standardized information about the elements of this namespace. This information is particularly important for aggregating Servers.

Typically, all Nodes of a companion specification are static and therefore the metadata shall describe them as static. This is done by setting all Numeric NodeIds to static (StaticNodeIdTypes). If you use different NodeIds (e.g. Strings), this needs to be adapted. If not all Nodes are static, it needs to be adapted as well. Static NodeIds mean, that the same Node is used in all servers, e.g. for TypeDefinitions or entry points like the “Root” Object of the base specification. Not static Nodes would be Nodes providing server-specific information (e.g. typically all the instances based on the TypeDefinitions of a companion specification) or other dynamic behaviour (e.g. a standardized Method that adds or removes something from a server).

Table 39 defines the namespace metadata for this document. The Object is used to provide version information for the namespace and an indication about static Nodes. Static Nodes are identical for all Attributes in all Servers, including the Value Attribute. See OPC 10000-1 for more details.

The information is provided as Object of type NamespaceMetadataType. This Object is a component of the Namespaces Object that is part of the Server Object. The NamespaceMetadataType ObjectType and its Properties are defined in OPC 10000-1.

The version information is also provided as part of the ModelTableEntry in the UANodeSet XML file. The UANodeSet XML schema is defined in OPC 10000-6.

Table 62 – NamespaceMetadata Object for this Document

| Attribute | Value |  |  |
| --- | --- | --- | --- |
| BrowseName | http://opcfoundation.org/UA/<short name>/ |  |  |
| Property |  | DataType | Value |
| NamespaceUri |  | String | http://opcfoundation.org/UA/<short name> |
| NamespaceVersion |  | String | X.YY |
| NamespacePublicationDate |  | DateTime | YYYY-MM-DD |
| IsNamespaceSubset |  | Boolean | False |
| StaticNodeIdTypes |  | IdType [] | {Numeric} |
| StaticNumericNodeIdRange |  | NumericRange [] | Null |
| StaticStringNodeIdPattern |  | String | Null |

Note: The IsNamespaceSubset Property is set to False as the UaNodeSet XML file contains the complete Namespace. Servers only exposing a subset of the Namespace need to change the value to True.

## Handling of OPC UA Namespaces

Namespaces are used by OPC UA to create unique identifiers across different naming authorities. The Attributes NodeId and BrowseName are identifiers. A Node in the UA AddressSpace is unambiguously identified using a NodeId. Unlike NodeIds, the BrowseName cannot be used to unambiguously identify a Node. Different Nodes may have the same BrowseName. They are used to build a browse path between two Nodes or to define a standard Property.

Servers may often choose to use the same namespace for the NodeId and the BrowseName. However, if they want to provide a standard Property, its BrowseName shall have the namespace of the standards body although the namespace of the NodeId reflects something else, for example the EngineeringUnits Property. All NodeIds of Nodes not defined in this document shall not use the standard namespaces.

Table 40 provides a list of mandatory and optional namespaces used in an <title> OPC UA Server.

Table 63 – Namespaces used in a <title> Server

| NamespaceURI | Description | Use |
| --- | --- | --- |
| http://opcfoundation.org/UA/ | Namespace for NodeIds and BrowseNames defined in the OPC UA specification. This namespace shall have namespace index 0. | Mandatory |
| Local Server URI | Namespace for nodes defined in the local server. This namespace shall have namespace index 1. | Mandatory |
| http://opcfoundation.org/UA/DI/ | Namespace for NodeIds and BrowseNames defined in OPC 10000-100. The namespace index is Server specific. | Mandatory |
| http://opcfoundation.org/UA/<title>/ | Namespace for NodeIds and BrowseNames defined in this document. The namespace index is Server specific. | Mandatory |
| Vendor specific types | A Server may provide vendor-specific types like types derived from ObjectTypes defined in this document in a vendor-specific namespace. | Optional |
| Vendor specific instances | A Server provides vendor-specific instances of the standard types or vendor-specific instances of vendor-specific types in a vendor-specific namespace. It is recommended to separate vendor specific types and vendor specific instances into two or more namespaces. | Mandatory |

Table 41 provides a list of namespaces and their indices used for BrowseNames in this document. The default namespace of this document is not listed since all BrowseNames without prefix use this default namespace.

Table 64 – Namespaces used in this document

| NamespaceURI | Namespace Index | Example |
| --- | --- | --- |
| http://opcfoundation.org/UA/ | 0 | 0:EngineeringUnits |
| http://opcfoundation.org/UA/DI/ | 2 | 2:DeviceRevision |
| http://opcfoundation.org/UA/Dictionary/IRDI/ | 3 | 3:0112/2///61987#xzx608 |

(normative) 

<Title> Namespace and mappings

Namespace and identifiers for <Title> Information Model

This appendix defines the numeric identifiers for all of the numeric NodeIds defined in this document. The identifiers are specified in a CSV file with the following syntax:

<SymbolName>, <Identifier>, <NodeClass>

Where the SymbolName is either the BrowseName of a Type Node or the BrowsePath for an Instance Node that appears in the specification and the Identifier is the numeric value for the NodeId.

The BrowsePath for an Instance Node is constructed by appending the BrowseName of the instance Node to the BrowseName for the containing instance or type. An underscore character is used to separate each BrowseName in the path. Let’s take for example, the <type> ObjectType Node which has the <property> Property. The Name for the <property> InstanceDeclaration within the <type> declaration is: <type>_<property>.

A NamespaceURI follows the convention: http://opcfoundation.org/UA/<short name>/.

<short name> is described in 14.

.

Note that NamespaceURIs are NOT live URLs. Text in the specification should not suggest that they are.

The NamespaceUri for all NodeIds defined here is http://opcfoundation.org/UA/<short name>/

File Locations

The location of any version dependent files follow this convention:

http://opcfoundation.org/UA/schemas/<short name>/<version>/<file name>

The <short name> is the same as specified in the NamespaceURI;

The <version> is a number with the form #.# or #.##;

The location of the version independent files are the same but with the <version> omitted.

e.g. http://opcfoundation.org/UA/schemas/<short name>/<file name>

File Names

NodeIds: 	Opc.Ua.<short name>.NodeIds.csv or <short name>.NodeIds.csv

NodeSet: 	Opc.Ua.<short name>.NodeSet.xml or <short name>.NodeSet.xml;

Any other files should have a prefix that provides context when the file is downloaded in a browser.

All published files must be added to GitHub https://github.com/OPCFoundation/UA-Nodeset

This can be done by creating a mantis issue in the “NodeSets, XSDs and Generated Code” project:

https://opcfoundation-onlineapplications.org/mantis/main_page.php

The files should be attached to the mantis issue.

If the NodeSet was generated with the Opc.Ua.ModelCompiler the design file should be attached as well.

The CSV released with this version of the specification can be found here:

http://www.opcfoundation.org/UA/schemas/<short name>/1.0/NodeIds.csv

NOTE    The latest CSV that is compatible with this version of the specification can be found here:

http://www.opcfoundation.org/UA/schemas/<short name>/NodeIds.csv

A NodeIds.csv file is not mandated but recommended.

It contains a flat list of NodeIds with unique names and can be used instead of a full NodeSet if only such NodeId constants for a programming environment are needed.

A computer processible version of the complete Information Model defined in this document is also provided. It follows the XML Information Model schema syntax defined in OPC 10000-6.

The Information Model Schema for this version of the document (including any revisions, amendments or errata) can be found here:

http://www.opcfoundation.org/UA/schemas/<short name>/1.0/Opc.Ua.<short name>.NodeSet2.xml

NOTE    The latest Information Model schema that is compatible with this version of the document can be found here:

http://www.opcfoundation.org/UA/schemas/<short name>/Opc.Ua.<short name>.NodeSet2.xml

_____________

—————————